RTL_EXERCISE_1 BOUND FLASHER

Author. Date; 2022/03/04. Version; 1.1.

Contents

1. Interface	2

2. Functional implementation.	3

3. Internal implementation.	5

3.1. Overall.	5

3.2. State Machine	6

4. History	7

# 1. Interface

16 1 1 16 flick rst clk bound_flasher_sys LED

Figure 1: the figure of Bound Flasher System

| Signal | Width | In/Out | Description |
| --- | --- | --- | --- |
| flick | 1 | In | Determine module’s operation, depend on current state |
| clk | 1 | In | Master clock signal, for apply pulse for module purpose |
| rst | 1 | In | To reset the module |
| LED | 16 | Out | 16 single LEDs in 1 column |

Table 1: Description of signals in Bound Flasher

# 2. Functional implementation.

- Implement a 16-bits LEDs system

- System’s Operation base on three input signals

  - Reset

  - Clock

  - Flick

- The system specification

- Clock signal is provided for system inspire of function status. The function operate state’s transition at positive edge of the clock signal.

- Reset signal:

- LOW-ACTIVE Reset = 0: System is restarted to Initial State.

- HIGH-ACTIVE Reset = 1: System is started with initial state.

- Flick signal: special input for controlling state transfer.

- At the initial state, all lamps are OFF. If flick signal is ACTIVE, the flasher start operating:

- The lamps are turned ON gradually from lamp[0] to lamp[5].

- The lamps are turned OFF gradually from lamp[5] (max) to lamp[0] (min).

- The lamps are turned ON gradually from lamp[0] to lamp[10].

- The lamps are turned OFF gradually from lamp[10] (max) to lamp[5] (min).

- The lamps are turned ON gradually from lamp[5] to lamp[15].

- The lamps are turned OFF gradually from lamp[15] to lamp[0], return to initial state.

At each kickback point (lamp[5] and lamp[10]), if flick signal is ACTIVE, the lamps will turn OFF gradually again to the min lamp of the previous state, then continue operation as above description.

For simple, kickback point is considered only when the lamps are turned ON gradually, except the first state.

- Some insulations:

- When flick = 0 at kickback points

- When flick = 1 at kickback points (lamp[10])

# 3. Internal implementation.

## 3.1. Overall.

Figure 3.1: Block diagram of Bound Flasher

Module; Signal; Description. clk_divider; input clk input rst output new_clk. LED_counter; input clk input up_down input rst [3:0] input start_num [3:0] input end_num [3:0] output counter_out output check. LED_display; [3:0] input counter_out [15:0] output led_out; Converting 4 bits number (represent for on-light leds) to 16 bits displayed on single LED, start from bit 0.. trigger_creating; input enable input clk input rst output trigger; To create trigger for Mode_memory, Mode_memory, and Mode_processing.. Mode_memory; [2:0] input mode input trigger input rst [2:0] output prev_mode; To store the previous mode value.. maxNum_Memory; input trigger input rst [3:0] input maxNum [3:0] output prev_maxNum; To store the previous maxNum value.. mode_processing; [2:0] input prev_mode input check input flick input rst [2:0] output mode; If check pulse up, data from flick & prev_mode will be combined to caculate mode (new mode).. convert_mode_to_maxNum; [2:0] input mode [3:0] output maxNum; maxNum represent for number of on-light corresponding to mode.. compare; [3:0] input a [3:0] input b output larger_smaller.

Table 3.1: Block diagram of Bound Flasher Description

## 3.2. State Machine

Figure 3.2: State Machine of Bound Flasher

| Variable Name | Description |
| --- | --- |
| flick |  |
| posedge |  |

Table 3.2: Variable name of State machine

| State name of State machine | Description |
| --- | --- |
| Initial | All lamps are OFF |
| Mode 1 | Turn ON gradually from lamp [0] to lamp [5]. |
| Mode 2 | Turn OFF gradually from lamp [5] (max) to lamp [0] (min). |
| Mode 3 | Turn ON gradually from lamp [0] to lamp [10]. |
| Mode 4 | Turn OFF gradually from lamp [10] (max) to lamp [5] (min) |
| Mode 5 | Turned ON gradually from lamp [5] to lamp [15] |

Table 3.3: State name of State machine

# 4. History

| Date | Author | Modified part | Description |
| --- | --- | --- | --- |
| 06/03/2022 | Lê Hoàng Minh Tú Nguyễn Trọng Nhân | Interface | - New creation & complete interface & block diagram. - Block diagram of Bound Flasher Description (without detailed decripstion) |
| 07/03/2022 | Lê Hoàng Minh Tú | block diagram, block diagram description | - Add rst signal in block diagram - Fill Mode_memory & maxNum_Memory description |
| 8/3/2022 | Nguyễn Trọng Nhân | block diagram, block diagram description | - Fill LED_display & convert_mode_to_maxNum & mode_processing |
| 8/3/2022 | Nguyễn Thanh Sang | FSM + Explaination | -Fill Trigger_creating, State name of State machine description |